CPU

RAM

I/O

BUS

Schematic Diagram of a Computer

I/O

Input and Output devices are how information is gotten into and out from the computer.  It is how it communicates with the outside world

Input

Output

RAM

Random Access Memory, which means that the time to access any address takes the same time.

Consider memory as an array or list of bytes, mem[0],mem[1],…mem[G]

A byte is the smallest addressable space in RAM.  A byte is made up of 8 bits. There is an historical reason to choose 8 bits as a byte.  It is because of the storing of information.  (Hint:How many keys are there on the old keyboards for typewriters?).

A bit is the smallest storage unit on the computer. It holds two values, 1 or 0.  Historically this is because the first memory unit was a vacuum tube which had two physical states, on or off. Later it was replaced by magnetic core, which also had two physical states, positive and negative. Later when semi-conductor memory became the basis of RAM, it was given two states as well, high and low (voltage).

Part of the RAM is made up of Read Only Memory (or ROM), which is memory which cannot be modified.

CPU

Central Processing System is where the modifications and manipulations take place, the so called “brain” of the computer.

4 Types of Instructions

Data movement

Load

Store

Data manipulation

Add

Subtract

Multiply

Divide

And

Or

Not

Control instruction

If x < o

x  -x replace x by –x.

Bneg, this states if the contents of an address is negative, to skip the next instruction and jump to an address given by the instruction

Branch, this states to go find the next instruction at a different address. This is a sort of goto.

Misc

Print

Halt

Von Neumann Model of a Computer

Fetch		Fetch the next instruction from memory where the Program Counter (PC) is pointing to and bring it into the CPU and store it in the Instruction Register (IR).  Then increment the PC to point to the next instruction.

Decode	Decode the instruction in the IR; meaning find out what the instruction needs to execute, and if it is not in the CPU, bring it into the CPU.

Execute	Execute the instruction; meaning to carry out the instruction. E.g., if it is an ADD instruction, do the necessary addition.

1000:

LOAD  104

1000

PC					IR

FETCH:      IR  MEM[PC]

PC  PC + 1

DECODE:   READS THE OPCODE (LOAD, STO) AND GET WHAT IS NEEDED INTO THE CPU

EXECUTES:  EXECUTES, for example,  IT DOES THE ADDITION

A picture of a CPU

C = A + B

Symbol Table

| Name | Type | Address |
| --- | --- | --- |
| a | int | 104 |
| b | int | 108 |
| c | int | 112 |

104  108  112						1000  1001  1002

LOAD 104

ADD    108

2

3

STO     112

4

PC						IR

1000

CPU                            Ram

| State | PC | IR | MAR | MBR | ACC | 104 | 108 | 112 | 1000 | 1001 | 1002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1000 | ? | ? | ? | ? | 2 | 3 | 4 | Load 104 | Add 108 | Sto 112 |
| Fetch |  |  |  |  |  |  |  |  |  |  |  |